EXERCISE: A Quick-Look Evaluation

| Objective | You have been asked to perform a quick ‘back-of-the-envelope’ evaluation of the blocks being offered in the Bonanza Basin, offshore Somewhereia. Is there enough potential that our company should take a serious look? |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Reservoir Quality Use Figure 7 to indicate where the reservoir is expected to be excellent, intermediate, or poor based on the model shown in Figure 6b. You can use three colors or three line patterns to indicate the areal distribution of reservoir quality. |

Figure 7.  Map of reservoir quality based on simple modeling.

Continued on next page

Exercise: A Quick-Look Evaluation  continued

| Step | Action |
| --- | --- |
| 2 | Depth of Source For our purpose, we will assume that the source is everywhere 400 meters deeper than the top of the reservoir. Based on this assumption, use Figure 8 to indicate where you expect oil and gas generation to occur based on the model shown in Figure 6a. (Hint: Subtract 400 m from the depths shown in Figure 6a to compensate for the 400 m depth difference between the reservoir and the source intervals.) |

Figure 8.  Map of source maturity (oil/gas generation) based on simple modeling.

Exercise: A Quick-Look Evaluation  continued

QUESTIONS:

- Are there any blocks that look promising, i.e., that might have traps that could have significant amounts of oil or gas?  Which ones?

- The seismic lines suggest that large rotated fault blocks should be present.

- We have a potential sandstone reservoir with excellent to good predicted porosity over much of the area.

- We have a source that would generate oil and it is mature over three of the northern blocks and further to the north.

- On the seismic line, the seal unit looks like it is present over the open blocks.

There does not appear to be a major flaw in any one of the play elements given our quick look.  Perhaps blocks 5, 7, and 8 are more favorable.

- Are there any blocks that hold very little potential? Which ones?

The southern tier of blocks (1, 2, 3, and 4) does not have mature source rocks. We could still have HC migration from downdip (north) to updip traps (south), so these blocks could still hold good potential.

- In your opinion, should our company form a team to do more evaluations?

YES                                      NO

YES, since we do not have evidence that one of the play elements will not work in these blocks.  It is worth spending more time, money and people time to do more careful mapping and analysis, especially since we have learned that more seismic data, a regular 2D grid, is available for purchase.