Exercise 2:  A Quick-Look Evaluation

| Objective | You have been asked to perform a quick ‘back-of-the-envelope’ evaluation of the blocks being offered in the Bonanza Basin, offshore Somewhereia. Is there enough potential that our company should take a serious look? |
| --- | --- |

| Materials | - Exercise instructions (which includes figures) - Colored pencils |
| --- | --- |

| Introduction | Our regional geology team has developed a list of high-potential basins and the Bonanza Basin is high on that list. We have learned that eight blocks in the Bonanza Basin have just been put up for bid. The bids are due in six months. Our company is always looking for great opportunities. Before management agrees to have a team work on the area, they would like a quick look on how profitable these blocks might be. Thus, you have been commissioned to “test the waters.” With only a little time (about a week) and not much data, you are to make a recommendation on whether a more thorough evaluation (i.e. more people working on the project for ~5 months) is worth the effort. |
| --- | --- |

| Available Data | A check of in-house and publically available literature reveals that a well was drilled on the shelf, just updip from the open blocks. There are also a few seismic lines in the area. Both the seismic lines and well information are in the company data library. Figure 1 shows the location of the well and the available seismic lines. The well information, from a scouting service, is sparse (Figure 2). However, the well had a good nearshore (beach) sandstone that could form a high-quality reservoir. A good marine shale lies above the sandstone. It should provide an adequate seal. Source rocks that should be able to yield oil are present in the well about 400 m below the top of the reservoir. At the location of the well this source interval is immature (not buried deeply enough to start generating HCs). The conclusion from the well information is that there is a potential reservoir, source, and seal. The open blocks do hold promise if: (1) the source is more deeply buried (more ‘cooked’), (2) the sandstone unit extends far enough out into the basin, and (3) the sandstones maintain good porosity further out into the basin. |
| --- | --- |

Continued on next page

Exercise 2:  A Quick-Look Evaluation  continued

Bonanza Basin

Figure 1.  Index map showing the location of the A-1 well, the available seismic lines, and the open blocks.

Continued on next page

Exercise 2:  A Quick-Look Evaluation  continued

Nearshore Sandstones – a Potential Reservoir

Marine Shale – a Potential Seal

Organic-Rich Shales – a Potential Source

Well A-1

250 m

650 m

350 m

0 m

800 m

Sands contain brine – No HCs

Good HC source characteristics

Immature – not generating oil or gas

Figure 2.  Simplified stratigraphy for the A-1 well that is based on information from a scouting service.  Note that the source interval lies 400 m below the top of the potential sandstone reservoir.

Continued on next page

Exercise 2:  A Quick-Look Evaluation  continued

| Available Data Continued | One of the 2D seismic lines passes near the well location (Figure 3). The potential source, reservoir, and seal units extend out to the open blocks, then they are cut away by a major erosional event in the early Tertiary. Although the units extend out into the basin, sedimentary facies may change downdip (e.g., the sandstone may grade into a siltstone and then shale). The reservoir is also deeper and we expect porosity to decrease with depth. On the positive side, there are some rotated fault blocks that could provide large traps. |
| --- | --- |

Figure 3.  A 2D seismic line that passes near the A-1 well location and NNE out into the basin through block 2 and block 6.

Continued on next page

Exercise 2:  A Quick-Look Evaluation  continued

| Available Data Continued | A study done at a university provides insights into how the Bonanza Basin formed. This study indicates that it is a divergent (pull-apart) margin with rifting during the Late Jurassic. The A-1 well is on normal continental crust, but most of the open acreage is on thinned continental or oceanic crust (Figure 4). This means higher heat flow and deeper burial for the sediments in the open blocks than at the A-1 well location. The source interval may have been heated enough for oil and gas to be generated and expelled. |
| --- | --- |

Bonanza Basin

Figure 4.  A map of crustal type in our area of interest, based on an academic study of the greater Bonanza Basin.

Continued on next page

Exercise 2:  A Quick-Look Evaluation  continued

| Available Data Continued | You perform a quick interpretation of the top of the potential reservoir unit on all of the available seismic lines. You use a simple relationship to convert from seismic two-way time to depth. Figure 5 shows your results (depths are in kilometers). Over the open blocks, the potential reservoir varies in depth from about 500 m to 6000 m. |
| --- | --- |

Bonanza Basin

Figure 5.  Top of the potential reservoir unit based on a quick interpretation of the available seismic lines and a simple time-to-depth relationship.  (NOTE: This map is adequate for a quick-look. More careful work would be required for prospecting).

Continued on next page

Exercise 2:  A Quick-Look Evaluation  continued

| Available Data Continued | You have one more ‘trick’ to apply to your analysis. Using the well information, you can model how the source rock would mature in terms of generating HCs as a function of depth. You can also model how reservoir porosity would decrease with depth. The results of this modeling are shown in Figure 6. |
| --- | --- |

Figure 6.  Modeling of (A) source maturity (oil and gas generation) and (B) reservoir porosity as a function of depth.

Continued on next page

Exercise 2:  A Quick-Look Evaluation  continued

| Step | Action |
| --- | --- |
| 1 | Reservoir Quality You have approximate depths to the top of the reservoir (Figure 5) and a rough model of reservoir quality (porosity) as a function of depth (Figure 6b). Figure 7 is an uncolored version of Figure 5 showing the depth to the top of the reservoir. Use Figure 7 to indicate where the reservoir is expected to be excellent, intermediate, and poor based on the model shown in Figure 6b. You can use three colors or three line patterns to indicate the areal distribution of reservoir quality. |

Figure 7.  Map of reservoir quality based on simple modeling.

Continued on next page

Exercise 2:  A Quick-Look Evaluation  continued

| Step | Action |
| --- | --- |
| 2 | Depth of Source Figure 6a relates oil and gas generation as a function of the depth of the source interval. You have a map to the top of the reservoir, but not one for the top of the source. In the well the top of the source is 400 m deeper than the top of the reservoir. The seismic line (Figure 3) shows that the intervals thicken slightly to the north. For our purpose, we will assume that the source is everywhere 400 meters deeper than the top of the reservoir. Using this assumption, use Figure 8 to indicate where you expect oil and gas generation to occur based on the model shown in Figure 6a. (Hint: Subtract 400 m from the depths shown in Figure 6a to compensate for the 400 m depth difference between the reservoir and the source intervals.) |

Figure 8.  Map of source maturity (oil/gas generation) based on simple modeling.

Exercise 2:  A Quick-Look Evaluation  continued

QUESTIONS:

- Are there any blocks that look promising, i.e., that might have traps that could have significant amounts of oil or gas? Which ones?

- Are there any blocks that hold very little potential? Which ones?

- In your opinion, should our company form a team to do more evaluations?

YES                                      NO