Exercise 3:  Play Elements

| Objective | Use seismic interpretation products in the form of a series of maps to evaluate the HC potential of eight blocks that are open to bidding in the Bonanza Basin. |
| --- | --- |

| Materials | - Exercise instructions (which includes maps) - Color pencils - Tracing paper |
| --- | --- |

| Introduction | Our regional geology team has developed a list of high-potential basins and the Bonanza Basin is high on that list. Now eight (8) blocks are up for bid in this basin. A team of five (5) people is tasked with evaluation of these eight (8) blocks. There is a well just outside the open blocks that encountered a good reservoir and a good source interval. Unfortunately, the well was dry. At the well location the source interval is immature (no hydrocarbon (HC) generation). The team generated a series of maps based on the well data and a grid of 2D seismic data (more data than in Exercise 2). The team also modeled source maturation and reservoir porosity as a function of burial depth. You will use these maps to decide: - Which blocks merit further work leading up to a bid - Good - Which blocks show little HC potential - Bad - Which blocks are in still worth of consideration - Possible Figure 1 shows the location of the eight (8) blocks, the A-1 well, and the grid of 2D seismic lines. |
| --- | --- |

Continued on next page

Exercise 3:  Play Elements  continued

Bonanza Basin

Somewhereia

Block

1

Block

5

Block

4

Block

3

Block

2

Block

8

Block

7

Block

6

Well A-1

Figure 1.  Index map showing the location of the A-1 well, the 2D seismic grid, and the eight (8) open blocks.

Continued on next page

Exercise 3:  Play Elements  continued

| Step | Action |
| --- | --- |
| 1 | RESERVOIR The seismic interpreter mapped the major faults and horizons. He focused on the interval that correlates to the potential sandstone reservoir in the A-1 well. S/he interpreted gross depositional environments, based on the thickness, 3D geometry, and seismic reflection characteristics of this interval. Given these, s/he inferred the likely distribution of sandstones. S/He mapped two sand-prone facies: nearshore sandstones (beaches), similar to those encountered in the A-1 well, and deepwater channel and fan sandstones (Figure 2). S/He also saw diapiric features that s/he interpreted as related to salt. |
| 2 | TRAP The seismic interpretation also resulted in maps that show a number of possible structural traps. Some are fault-bounded highs, while other are four (4)-way closures due to the faulting and folding of the region. There is a salt ridge in the northern row of blocks that leads to a number of salt-supported highs with four (4)-way closures. On the map in Figure 3, potential traps are indicated by the blue-green polygons. |
| 3 | SOURCE MATURITY Our geochemist developed a model of source maturation and HC generation based on burial history and thermal gradients. Figure 4 shows areas where his/her model indicates the source should generate oil and gas. In the deepest areas, HC generation has ceased (the source is fully spent). |

Continued on next page

Exercise 3:  Play Elements  continued

Bonanza Basin

Figure 2.  Map shows interpreted depositional environments and inferred lithologies for the primary reservoir interval.

Continued on next page

Exercise 3:  Play Elements  continued

Bonanza Basin

Figure 3.  Map shows potential structural traps in the Bonanza Basin. Note the salt ridge and anticlines in the northern row of blocks.

Continued on next page

Exercise 3:  Play Elements  continued

Figure 4. Map shows where oil and gas are currently generated, based on a model that incorporates burial, heat flow, and HC kinetics. The A-1 well offered some calibration for the model.

Continued on next page

Exercise 3:  Play Elements  continued

| Step | Action |
| --- | --- |
| Bonanza Basin 4 | MIGRATION PATHWAYS Defining HC migration paths is not easy based on seismic data. As a proxy, geologists and geophysicists analyze a seismic-based horizon for buoyancy-driven fluid flow. They determine local high points (blue diamonds) and define drainage polygons (blue dashed lines). In addition, they construct flow paths by finding the direction of maximum dip up to each culmination. Figure 5 shows these types of flow paths based on the top of the reservoir horizon. |

Figure 5.  Map shows possible buoyancy-driven flow paths based on the structure (shape) of the top reservoir horizon.

Continued on next page

Exercise 3:  Play Elements  continued

| Step | Action |
| --- | --- |
| 5 | TASK: You are interested in where there is a favorable juxtaposition of the five (5) play elements: source, reservoir, trap, seal and migration. For this exercise, we will assume a good seal overlies the reservoir everywhere. Use the tracing paper to determine where there is a favorable combination of play elements. The instructor will share a valuable tip on how to proceed. Based on your analysis, categorize each block into “Good,” “Bad,” or “Possible.” You can circle the appropriate word for each block below. Block 1 Good Bad Possible Block 2 Good Bad Possible Block 3 Good Bad Possible Block 4 Good Bad Possible Block 5 Good Bad Possible Block 6 Good Bad Possible Block 7 Good Bad Possible Block 8 Good Bad Possible |