Exercise 5a:  An Inclined Reflector

| Objective | Use a simple figure of an inclined reflector to determine where the seismic response would be plotted (displayed) on a seismic section – before correction. |
| --- | --- |

| Materials | - Exercise instructions - A compass (or string and thumb tack) |
| --- | --- |

| Introduction | One result of the seismic method is that inclined reflectors are not correctly positioned on a seismic line unless a correction is applied in the processing stream, namely seismic migration. Seismic ray paths are perpendicular to the reflecting surface, but their arrival time is plotted vertically beneath the source-receiver midpoint. Figure 1 (next page) represents a zero-offset seismic line (the source and receiver are in the same place with no offset). As shown for location 1, the ray path is perpendicular to the surface (the shortest distance). However, the seismic event is plotted directly beneath the surface location. Plot where the seismic event would be displayed on a seismic section for locations 1 through 8. |
| --- | --- |

Continued on next page

Exercise 5a:  An Inclined Reflector  continued

1

2

3

4

6

5

Where does the reflection lie?

90º

Figure 1.  Given this inclined reflection surface, construct where the reflection would appear on an uncorrected (unmigrated) seismic line.

| Step | Action |
| --- | --- |
| 1 | Draw lines from the surface locations to the reflector that are perpendicular (90º) to the reflector. |
| 2 | Draw a vertical line down from each surface position. |
| 3 | Use a compass to draw an arc such that the compass point is on the surface location and the pencil point starts on the place where the ray path is perpendicular to the reflector (line shown in Figure 1). Then swing an arc to the position directly below the surface shot point. |

Continued on next page

Exercise 5a:  An Inclined Reflector  continued

After you complete these steps for all the locations, consider:

True or False:

- When you see an inclined reflector on uncorrected (unmigrated) seismic data, the surface that caused it has a greater dip.

TRUE  or  FALSE

- When you see an inclined reflector on uncorrected (unmigrated) seismic data, the surface that caused is located further updip.

TRUE  or  FALSE