Exercise 5b:  An Anticline

| Objective | Use a simple figure of an anticline to determine where the seismic response would be plotted (or displayed) on a seismic section – before applying a correction. |
| --- | --- |

| Materials | - Exercise instructions - A compass (or string and thumb tack) |
| --- | --- |

| Introduction | Anticlines and synclines are not positioned correctly on a seismic line if the seismic data are not migrated during data processing. This is because the recorded seismic energy reflects perpendicularly to the reflecting surface, but their arrival time is plotted vertically beneath the source-receiver midpoint. As in the previous exercise, Figure 1 (next page) represents a zero-offset seismic line (no offset between the source and the receiver). The ray paths that are perpendicular to the surface are drawn for you. Once again, the seismic events plot directly beneath the surface location. Plot where the seismic event would be displayed on a seismic section for locations 1 through 6. |
| --- | --- |

Continued on next page

Exercise 5b:  An Anticline  continued

3

4

2

1

7

8

6

5

Subsurface

Interface

Figure 1.  	Given this reflection surface with the shape of a simple anticline, construct how it would appear on an uncorrected (unmigrated) seismic line.

| Step | Action |
| --- | --- |
| 1 | Draw a vertical line down from each surface position. |
| 2 | Use a compass to draw an arc in which the compass point is on the surface location and the pencil on the place where the ray path (already drawn) intersects the reflector. Then draw an arc to the position directly below the shot location. |
| 3 | The seismic event plots where the arc intersects a vertical line drawn down from the surface shot point. Mark the intersections with a small, bold circle. |

Continued on next page

Exercise 5b:  An Anticline  continued

After you complete these steps for all the locations, consider:

True or False:

- The crest of an anticline will appear to be too shallow on the seismic line.

TRUE  or  FALSE

- The slopes of the anticline will appear to have lower dip than the actual subsurface interface.

TRUE  or  FALSE

- The area of an anticline will be underestimated based on unmigrated seismic data.

TRUE  or  FALSE