Exercise 5c:  A Syncline

| Objective | Use a simple figure of a syncline to determine where the seismic response would be plotted (or displayed) on a seismic section – before applying a correction. |
| --- | --- |

| Materials | - Exercise instructions - A compass (or string and thumb tack) |
| --- | --- |

| Introduction | Anticlines and synclines are not positioned correctly on a seismic line if the seismic data are not migrated during data processing. This is because the recorded seismic ray paths are ones that are perpendicular to the reflecting surface, but their arrival time is plotted vertically beneath the source-receiver midpoint. As in the previous two exercises, Figure 1 (next page) represents a zero-offset seismic line (no offset between the source and the receiver). Only a few ray paths that are perpendicular to the surface are shown. Note that for shot #4 there are three (3) paths that are perpendicular to different locations along the subsurface interface. Once again, the seismic events plot directly beneath the surface location. Plot where the seismic event would be displayed on a seismic section for locations 1 through 6. |
| --- | --- |

Continued on next page

Exercise 5c:  A Syncline  continued

Figure 1.  	Given this reflection surface with the shape of a simple syncline, construct how it would appear on an uncorrected (unmigrated) seismic line.

| Step | Action |
| --- | --- |
| 1 | Finish drawing the ray paths on Figure 1, lines that start at the shots and hit the interface at 90°. |
| 2 | Draw a vertical line down from each surface position. |
| 3 | Use a compass to draw an arc – compass point on the surface location and pencil point on the place where the ray path intersects the reflector. |
| 4 | The seismic event plots where the arc intersects the vertical line. Mark the intersections with a small, bold circle. |

Continued on next page

Exercise 5c:  A Syncline  continued

After you complete these steps for all the locations, consider:

True or False:

- The low point of the syncline will appear to be too shallow on the seismic line.

TRUE  or  FALSE

- The slopes of the syncline will appear to have lower dip than the actual subsurface interface.

TRUE  or  FALSE

- The area of a syncline will be underestimated based on unmigrated seismic data.

TRUE  or  FALSE