Exercise: Shelf Margin Predictions from Seismic

| Objective | Use interpreted horizons on eight (8) seismic lines to map shelf margin environments of deposition (EODs) and predict the location of the best sands for hydrocarbon (HC) reservoirs. |
| --- | --- |

| Materials | - Exercise instructions - Eight (8) seismic lines with interpreted horizons - Four (4) copies of the base map (one for each sequence) - Pencils |
| --- | --- |

| Introduction | You have learned about the environments where reservoir-quality sands can be deposited; one of these is a shelf margin. You were introduced to the terminology of Rich (1951): undaform, clinoform and fondoform (Figure 1). Where we have had shelfal progradation, we can use geometries seen on seismic lines to make some EOD predictions. The data we will use were acquired and processed in the 1970s. Data quality is quite poor by today’s standards. In addition, the scale of your lines is compressed to page size. In spite of these two facts, we can still use seismic geometries to make some interpretations. Here are some definitions for you to use: 1. Shallow: the sequence was deposited on the underlying shelf. It is quite thin and usually has parallel to slightly inclined reflections. This facies tends to thin to below seismic resolution updip; it often thickens in a downdip direction. 2. Undaform: is where the sequence thickens considerably as deposition extends beyond the previous shelf edge. The undaform extends downdip to where the final shelf edge for any particular depositional period exists. The top of the undaform was deposited near wavebase and this high-energy environment often leads to sandy lithofacies in the undaform. The reflections are normally inclined, dipping down in a basinward direction. |
| --- | --- |

Continued on next page

Exercise: Shelf Margin Predictions from Seismic cont

| Introduction continued | 3. Clinoform: the portion of the sequence just downdip from the final shelf edge. The interval thins as the paleobathymetry increases. As water depths increase, energy conditions decrease resulting in mostly silt and then shale deposition. The reflections are normally inclined, dipping down in a basinward direction. 4. Fondoform: the portion of the sequence that is basinward of the final toe (base) of the paleo-slope. Shales and pelagic clays are the expected lithologies during deltaic deposition. In general, the reflections are parallel to subparallel. The unit is often thin and can get below seismic resolution in a downdip direction. Figure 2 shows the features you are to map: 1. The boundary between the 'shallow' and the undaform 2. The boundary between the undaform and the clinoform 3. The boundary between the clinoform and the fondoform 4. The position where the sequence pinches out (if applicable). |
| --- | --- |

Shallow

Figure 1.  Subdivision of a prograding depositional package (Rich, 1951).

Continued on next page

Exercise: Shelf Margin Predictions from Seismic    cont

Figure 2.  Line C showing the geometric features for the blue to pink sequence.

| Step | Action |
| --- | --- |
| 1 | Examine the seismic lines noting the geometric features that define the shallow zone, the undaform, the clinoform, and the fondoform (see Figure 2 for an example). Decide on which of the four sequences you want to start mapping. |
| 2 | Fill out Table 1 with the appropriate information for the sequence you chose in Step 1. |
| 3 | After completing Table 1, mark the positions you recorded along each seismic line on the base map. Color-code each line segment, for example: 1. Shallow = orange 2. Undaform = yellow 3. Clinoform = brown 4. Fondoform = green 5. Sequence absent/below resolution (e.g., blue). |

Continued on next page

Exercise: Shelf Margin Predictions from Seismic    cont

Table 1.  Data for Mapping Seismic Facies

Sequence: ________  to  ___________

| LINE | Shot point of transition from Shallow to Undaform | Shot point of transition from Undaform to Clinoform | Shot point of transition from Clinoform to Fondoform | Shot point where the sequence pinches out |
| --- | --- | --- | --- | --- |
| A |  |  |  |  |
| B |  |  |  |  |
| C |  |  |  |  |
| D |  |  |  |  |
| E |  |  |  |  |
| F |  |  |  |  |
| G |  |  |  |  |

| Step | Action |
| --- | --- |
| 4 | Can you draw a smooth boundary connecting each of the 4 seismic facies boundaries on all the lines? If not, can you reinterpret the positions of the boundaries in Table 1 so that there is lateral consistency? Seismic facies mapping is highly iterative. The goal is a vertically and spatially consistent interpretation that is geologically reasonable. Depositional dip lines are easier to interpret than depositional strike lines. |

Continued on next page

Exercise: Shelf Margin Predictions from Seismic    cont

| Step | Action |
| --- | --- |
| 5 | Based on your map, 1. Where would you expect fluvial to coastal plain deposits? 2. Where would you expect nearshore sands and delta plain deposits? 3. What types of lithologies would you predict for the clinoform region? 4. Where do you think the best place to find reservoir-quality sands is? 5. If a well was drilled in the undaform region, describe the vertical facies change within the unit (from top to bottom). |

Continued on next page

Exercise: Shelf Margin Predictions from Seismic    cont

| Step | Action |
| --- | --- |
| 6 | Select a second depositional sequence for the same type of analysis. Repeat Steps 2 through 5 for your second sequence. Table 2 is provided for recording your observations for your second seismic sequence. |

Table 2.  Data for Mapping Seismic Facies

Sequence: ________  to  ___________

| LINE | Shot point of transition from Shallow to Undaform | Shot point of transition from Undaform to Clinoform | Shot point of transition from Clinoform to Fondoform | Shot point where the sequence pinches out |
| --- | --- | --- | --- | --- |
| A |  |  |  |  |
| B |  |  |  |  |
| C |  |  |  |  |
| D |  |  |  |  |
| E |  |  |  |  |
| F |  |  |  |  |
| G |  |  |  |  |