Map a Regional Unconformity

| Objective | To map a regional unconformity, called the Top of Latrobe (TOL) unconformity. You will use a regional 2D seismic data set. About half of the lines have interpretation. |
| --- | --- |

| Materials | - Exercise instructions - A basemap showing the location of the lines - 14 seismic lines (color, 11x17 inches) - Green marker pen - Highlighter pen |
| --- | --- |

| Introduction | In this exercise, you will map the TOL unconformity on eight (8) out of fourteen (14) seismic lines from a regional 2D survey; the other six (6) lines have been interpreted for you. You may be asked to do this as a part of a team of two (2) or three (3) people. Your goal is to locate and get a rough size estimate for large anticlines that are our potential traps. The TOL unconformity is easy to spot. Below the unconformity are strong reflections, some of which show erosional truncation. Above the unconformity are low amplitude reflections. It is difficult to mark the color seismic sections with anything but a marker. This means erasing is not possible once you draw a line. Since our goal at this point is to find major anticlines, we do not have to be very precise. Even if you are several reflections too high or too low on the TOL position, the huge structures will still stand out. |
| --- | --- |

Continued on next page

Map a Regional Unconformity   continued

| Step | Action |
| --- | --- |
| 1 | Examine the basemap and note the layout of the lines. Note that the TOL unconformity has been marked for you as a green line on lines 2, 4, 6, 9, A and D. Examine the characteristics of this surface. - What is the amplitude strength and continuity of the reflections just below the TOL horizon? - How do the reflections just above the TOL differ? - The TOL horizon is what is commonly referred to as a ‘break-up unconformity’. There were tectonic movements as the area transitioned from rifting to sea floor spreading (drifting). Do you see evidence for faulting and erosion on the interpreted lines? For our purposes, we will only map the TOL horizon – not the faults. You can mark offsets in the TOL horizon with fault planes, but this is not necessary for the task at hand and our time is short. We will map faults later. |
| 2 | Start loop tying the horizon at line intersections. You might start with the intersection of Line A with Line 5. Get comfortable with the mechanics involved with folding the paper seismic sections and transferring a horizon from an interpreted line to an uninterpreted line at a line intersection. Here is a suggested procedure. Start by working Line B. - Transfer the TOL horizon from Line 2 to Line B - Transfer the TOL horizon from Line 4 to Line B - Transfer the TOL horizon from Line 6 to Line B - Transfer the TOL horizon from Line 9 to Line B - With these four (4) ‘control’ points as a start, draw a dotted line representing the TOL horizon along line B from the left side to the right side of the section. As you interpret more lines, use a colored pencil to update the basemap showing the lines that have been interpreted. |

Continued on next page

Exercise 11:  Map a Regional Unconformity   continued

| Step | Action |
| --- | --- |
| 3 | Do the same thing for Line C and Line E, but divide up the work (if working in teams). Those not working Lines C and E can start tying and interpreting Lines 1, 3, 5, 7 and 8. Continue working until you have an initial interpretation on all the lines. Remember to use a colored pencil to update the basemap as you interpret lines. |
| 4 | Review your initial interpretation (dotted lines). If you are satisfied that the TOL horizon ties fairly well at all the line intersections – GREAT! Remember that for our task we do not need to be very precise. You can achieve the goal of this exercise with some amount of inconsistencies. You can either leave the lines dotted, or quickly draw solid lines to mark your horizon. |
| 5 | The next step for an interpreter would be to generate a time structure map. You will do this later in the course. For now, we are just looking for large anticlines. View each line and note the position of large highs (e.g., on Line A from midway between Line 2 to ¾ of the way to line 4). Use the highlighter to mark the basemap. After you mark highs for each line, see if you can draw a polygon to define large structures. If there are intersecting lines that both show a high, great. If two parallel lines have a high and the perpendicular line does not negate an anticline, be optimistic and draw an oval. Since the line spacing is on the order of 10 km, some pretty big oil or gas fields could be hiding in this area. |