Exercise:  Prospect Scale Mapping

| Objective | In this exercise, you will use a grid of 2D seismic lines to map the top of Latrobe (TOL), which is the top of the primary reservoir, and generate a time-structure map. Once you have a time-structure map, you will be ready to consider where to locate a wildcat well (objective of the next exercise, Exercise 14). |
| --- | --- |

| Materials | - Exercise instructions - A basemap showing the location of the lines (2 copies) - 11 seismic lines (black & white and color, 11x17 inches) - Color pencils - Lead pencil - Eraser |
| --- | --- |

| Introduction | Again you will map the TOL unconformity. This time, however, you will be working a much smaller area and at a much higher level of detail. Interpretation is provided on five (5) of the eleven (11) lines. You should mark fault offsets (use the pencil), but do not worry about details just yet. You will work some faults in detail in a later exercise. Mark the TOL surface with a color pencil in a “white” half cycle (or loop). You can use any color (although the interpreted lines have the TOL horizon marked in red). You may want to erase your interpretation, if necessary, so draw your initial TOL horizon faintly. Use basemap #1 to keep track of your interpreted lines. Once you have mapped the TOL horizon, you will post two-way time (TWT) values down to this surface on basemap #2. Then you will contour the TWT values to get a time-structure map for the TOL surface. From your quick regional mapping in Blocks 31 and 32, what do you expect? Hopefully you are thinking of a large anticline elongate in a NE-SW direction. |
| --- | --- |

Continued on next page

Exercise: Prospect Scale Mapping   continued

| Step | Action |
| --- | --- |
| 1 | Examine basemap #1 and note the layout of the lines. The lines interpreted for you are in solid lines (lines 10, 18, 26, 60, 72). The lines you need to interpret are dotted. You can use a color pencil (or highlighter) to keep track of the lines with interpretation. |
| 2 | Examine basemap #2. The available 2D survey had 19 SW-NE lines (10-28) and 16 NW-SE lines (60-75). Unfortunately, to save a few bucks, our company did not purchase all the lines. On the seismic sections, the intersecting lines we purchased are shown as solid vertical lines; the intersections for the lines you do not have are shown as dashed lines. When measuring and posting two-way time (TWT) values, get time values at all the solid and dashed vertical lines. These locations are shown as small circles on basemap #2. This gives you a TWT value about every kilometer. |
| 3 | There are two (2) tasks for this exercise: (1) complete the TOL interpretation, and (2) read, post and contour time values on basemap #2. You can divide the work among your team members. Some can complete the TOL horizon interpretation. Even while they are doing this, others can start reading and posting time values from the interpreted lines on basemap #2. |

Continued on next page

Exercise: Prospect Scale Mapping   continued

| Step | Action |
| --- | --- |
| 4 | For those completing the TOL interpretation, you are experienced at tying lines (folding paper sections) to transfer a horizon from an interpreted line to an uninterpreted line. Given the amount of interpreted lines, it should not take too long to get a spatially consistent TOL surface. Use basemap #1 to keep track of your progress in interpreting and verifying the consistency of your interpretation (by checking the line intersections). |
| 5 | Some can start reading time values and posting the numbers at the appropriate location on basemap #2. Fold one of the seismic lines so that you can see the time scale at the extreme left or right of the line. This is your “ruler” for measuring time values. It is easier to record in units of milliseconds (e.g., 1234 ms) rather than seconds (1.234 sec). It is best to have two (2) people work together – one person reads the time and calls out the number while the other person writes the number on the basemap. Since the positions to read values are marked on the seismic sections (solid and dashed vertical lines) and on basemap #2 (small circles), you can ‘process’ a line quickly. Continue to read/post values until you have done all eleven (11) lines. |

Continued on next page

Exercise: Prospect Scale Mapping   continued

| Step | Action |
| --- | --- |
| 6 | The final step is to contour the time values so that you get a time-structure map. Use a contour interval of 100 ms. The contour map gives you a way to visualize the topography of the TOL surface. Since oil and gas migrate due primarily to buoyancy, HC molecules will collect at the high points on the TOL surface (assuming there is a trapping geometry and an overlying seal – a reasonable assumption given our understanding of the regional geology). Ideally, we would want to convert the time values (in milliseconds) to depth values (in meters or feet) by using velocity information. For our task at hand, we will assume that the time-structure map is close to what a depth-structure map would reveal. |