Exercise: Build a Fold Chart

| Objective | In this exercise, you will consider how the fold builds up for a line in a marine survey. |
| --- | --- |

| Materials | - Exercise instructions - Pencil |
| --- | --- |

| Introduction | You will consider a marine acquisition configuration that consists of: - A single source group - 16 receiver groups, each with a spacing of 25 meters - The source is to the right of the 16 receiver groups - There is a gap of 50 meters from the source to the near receiver - Locations on the surface are spaced 25 meters apart, which is both the shot and receiver spacing - The surface locations are numbered from 1 to 31 with location number 1 being on the left and 31 being on the right. The acquisition plan is to tow the source behind the ship with the first shot at location 18, the near receiver at location 16, and the far receiver at location 1. This is shown in Figure 1. For each successive shot, the boat sails down the line 25 meters, i.e., the shot moves from location 18 to 19, then 20, etc. This line extends many kilometers down the line (to the right). |
| --- | --- |

Figure 1.  The position of the first shot and the associated active receivers for shot 1. NOTE: there is no active receiver at position 17.

Continued on next page

Exercise: Build a Fold Chart   continued

| Step | Action |
| --- | --- |
| 1 | Figure 2 has a chart for showing how different shot-receiver pairs are acquired for 14 shots. The instructor will illustrate how to determine and mark midpoints for shot 1. Fill in the chart for shot 1 as it is demonstrated. |
| 2 | The instructor will illustrate how to determine and mark midpoints for shot 2. Fill in the chart for shot 2 as it is demonstrated. |
| 3 | Fold is the number of midpoints collected at each location. Since the equipment moves from left to right, the first few locations will have less than the maximum fold. The instructor will show how to start filling in the fold at the bottom of the chart. |
| 4 | Fill in the chart for all the other shots. |
| 5 | Complete the fold part of the chart. The shots and receivers would continue to roll (move) down the line. Once maximum fold is reached, it will remain constant until we approach the end of the line, where the fold would drop off (mirror image of the fold build up at the start of a line). |
| 6 | Answer the following questions: 1. What is the maximum fold that is achieved? _____ 2. At which surface location would the first reflection be recorded? __________ 3. Would this first reflection be from a near receiver or a far receiver? _________ 4. At which surface location would maximum fold first be achieved? __________ |

Continued on next page

Exercise: Build a Fold Chart   continued