Exercise: Determining Survey Area

| Objective | In this exercise, you are given a subsurface area to image. You will go through some simple calculations to determine the surface area required for a seismic survey that will fully image the subsurface target area. |
| --- | --- |

| Materials | - Exercise instructions - Pencil - Calculator |
| --- | --- |

| Introduction | When we define a portion of the subsurface that we want to image (Figure 1), we have to acquire a larger surface area than our subsurface target area. There are two (2) main factors to consider: (1) the surface area needed to capture diffractions around the edge of the image area, and (2) the surface area needed for a fold to build up (and drop off) as we start (or finish) each seismic line. You will look at these two (2) factors for a hypothetical 3D survey. |
| --- | --- |

Table I. Key Information for the Target

| Depth of target | 3050 m – 3350 m |
| --- | --- |
| Two-Way Time (TWT) of the target | 1000 ms - 1120 ms |
| Subsurface Image Area | 20 x 24 km |

| Step | Action |
| --- | --- |
| 1 | The maximum offset (the distance behind the boat for the last receiver) equals 1.5 times the maximum target depth. Offset Max = 1.5 * Depth Max = __________ |

Continued on next page

Exercise: Determining Survey Area   continued

Figure 1. Map showing the prospect and the area we want to image during a 3D seismic survey. The desired subsurface area is 20 x 24 km. The red line is the extent of an amplitude anomaly (bright spot). The black dashed lines show the 2D seismic lines used to generate the contour map. What surface area is required to get full imaging of this target region?

Continued on next page

Exercise: Determining Survey Area   continued

| Step | Action |
| --- | --- |
| 2 | The fold taper is the distance we have to acquire so that we can build up to the maximum fold. This is one (1) factor that makes the surface area larger than the image area at depth. The fold taper distance has to be added when the boat is beginning a swath of lines and when it is ending a swath of lines. Taper = 0.5 * Offset Max where: Taper = the survey taper (in meters) Offset Max = maximum offset distance (Step 1) Taper = ___________________ |
| 3 | Migration aperture is another factor that controls the survey's surface coverage. We need to capture reflections at the edges of the survey that have a maximum dip of 30°. This is so that any diffraction generated at the edge of the image area is recorded out to an angle of 30° (Figure 2), which is what most seismic migration algorithms use as the maximum angle for dipping events. Thus we have to extend the surface area so that we have the migration aperture "buffer" on all four (4) sides. Aperture = Depth Max * Tan θ where: Aperture = the survey aperture (in meters) Depth Max = maximum target depth Θ = 30° (or the steepest stratigraphic dip if this is > 30°) Hint: Tan 30° = 0.577 Aperture = ___________________ |

Continued on next page

Exercise: Determining Survey Area   continued

Figure 2.  At the east edge of the image area, a fault offset of the target horizon generates a diffraction (blue line). To properly image the horizon-fault contact, the diffraction energy out to 30° has to be recorded. The zero-offset raypath from the surface to the diffraction at 30° is shown by the red line. Aperture is the extra surface coverage we need on all four (4) sides, to capture the energy we need to perform a good seismic migration. (The red line is curved since velocity varies with depth in the overburden.)

| Step | Action |
| --- | --- |
| 4 | We get better imaging in regions of steep dip by shooting in the direction of maximum structural dip. Based only on this factor, we should shot the survey (check one): [ ] North – South [ ] East - West |

Continued on next page

Exercise: Determining Survey Area   continued

| Step | Action |
| --- | --- |
| 5 | Draw a rectangle on the map in Figure 1 that indicates the surface coverage required to properly image the geology within the indicated image area. Your rectangle does not have to be to scale. Hint: add the aperture distance on all four (4) sides, but the taper distance only on the two sides in the direction of shooting (north-south or east-west). |