Exercise 19: Mapping the Green & Blue Faults

| Objective | In this exercise, you will map two faults using lines and slices from the Barracouta 3D survey. The starting point for the Green fault is a vertical profile, where the fault is already interpreted. The starting point for the Blue fault is a time slice. You will tie lines and slices to build an areally consistent fault plane for each fault. |
| --- | --- |

| Materials | - Exercise instructions - 8 inlines, 4 with some interpretation - 8 crosslines, 5 with some interpretation - 8 displays of time slices (explained in the introduction) - Basemap - Color pencils |
| --- | --- |

| Introduction | You will do a little bit of fault interpretation, enough to learn the basic process. The starting point for each fault is different. For the Green fault, the initial interpretation is on inline 2800. For the Blue fault, we give you its fault trace on the 1300 ms time slice. There are three displays for the 900 ms time slice. The display on page 17 shows the reflection amplitude data. Page 18 shows the coherency-type data for the same time slice. For the display on page 18, white indicates a trace and its neighbor are almost identical in waveform shape, and the dark grey to black color indicates two neighboring traces are dissimilar. The presence of a fault trace may be indicated by dark lineations. On page 19, we have co-rendered the amplitude and coherency-type data, as discussed in lecture 19. The red-to-white-to-blue shades indicate the reflection amplitude at locations where the coherency value is high (above a user-defined threshold). Dark gray-to-black indicates where the coherency value is low (below the threshold) with no regard to the reflection amplitude at such points. All the other time slices (1000, 1100, 1200, 1300, and 1400) only have the co-rendering of the amplitude and coherency-type data. |
| --- | --- |

Continued on next page

Exercise 19: Mapping the Green & Blue Faults   continued

| Step | Action |
| --- | --- |
| 1 | Examine the three versions of time slice 900 (pages 17, 18 and 19). Do you understand what each display represents? If not, review lecture 19. |
| 2 | The Green fault has been interpreted on inline 2800 (page 11). Tie this inline to each time slice and transfer the Green fault pick from inline 2800 to each time slices (place a green dot on the time slices at the intersection points). After you have tied inline 2800 to each time slice, interpret (draw) the Green fault trace on the time slices. |
| 3 | Does the Green fault trace, as you mapped it on the time slices, intersect any other available inlines or crosslines? If so, transfer the Green fault from the time slices to the other inlines and crosslines (fold at intersections and transfer a green mark from the interpreted to the uninterpreted line/slice). As you interpret the Green fault on inlines and crosslines, use a color pencil (e.g., green) to mark (approximately) the extent of the fault on the basemap. This helps an interpreter understand which lines have been interpreted and which still need to be worked. Complete the interpretation of the Green fault by having a consistent fault plane on each inline, crossline, and time slice that it intersects. |

Continued on next page

Exercise 19:  Mapping the Green & Blue Faults   continued

| Step | Action |
| --- | --- |
| 4 | Note that the Blue fault is marked on time slice 1300. The Blue fault should be present on inlines 1500, 1600 and 1700. Tie the Blue fault from time slice 1300 to these three inlines. |
| 5 | Interpret the Blue fault on inlines 1500, 1600 and 1700. Mark the extent of the Blue fault on the basemap (blue line). |
| 6 | Use inlines 1500, 1600 and 1700 to mark and interpret the Blue fault trace on the other time slices. Can you get a Blue fault surface that is consistent on each inline, crossline, and time slice that it intersects? Keep the basemap updated with the extent of the Blue fault. |
| 7 | If you want, you can identify one or two other major faults either on the vertical lines (inlines and/or crosslines) or on the time slices. From your initial fault interpretations, tie your additional fault(s) to intersecting vertical lines/slices. Can you develop a 3D consistent fault plane(s) for your additional faults? |