Exercise 20a: Parasequence Correlation

| Objective | In this exercise, you will use Self Potential (SP) logs from five (5) wells to correlate several parasequences and parasequence sets. |
| --- | --- |

| Materials | - Exercise instructions - SP log cross section, partially interpreted |
| --- | --- |

| Introduction | The SP log is used to interpret sedimentary facies. This log is commonly displayed to the left of the well bore. For such displays, small deflections to the left indicate shales; larger excursions to the left indicate sands (see the cross section). Coastal plain and fluvial rocks vary in SP response depending on the sand-shale ratio. A log interpreter, using the SP and other logs, has interpreted the depositional environments. Her interpretation is indicated by the pattern-fill of the SP logs. The steps in interpreting the log cross section: 1. Select a datum, to minimize post-depositional effects (done). 2. Look for evidence of abrupt facies changes (e.g., shelf mudstones lying above fluvial sands) 3. Interpret parasequences and parasequence sets in each well individually 4. Correlate parasequence sets from one well to the next 5. Correlate parasequences from well to well. 6. Draw depositional facies boundaries from well to well and color- code the facies across the section. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | We have selected a datum for the cross section – a rapid deepening near the top of our zone of interest. Note how the interpreted depositional environments indicate a rapid deepening (i.e., in Well A there is a change from fluvial/estuarine sandstones below to shelfal muds above). |

Continued on next page

Exercise 20a: Parasequence Correlation   continued

| Step | Action |
| --- | --- |
| 2 | Look for other places in each well where there is an abrupt change in the interpreted depositional environments. Mark these with an unconformity symbol (a squiggly line). |
| 3 | A parasequence is characterized by a shoaling upwards pattern (e.g., shelfal muds transitioning into nearshore sands). A few shoaling upward parasequences are marked by short arrows (curving up and to the left) on the cross section. Mark a few parasequences with similar arrows for each well. A parasequence set is a series of shoaling parasequences in an overall shoaling upward package (e.g., a predominantly mudstone unit gradually becoming more shallow marine to perhaps fluvial.) Parasequence set boundaries often exhibit a significant change in depositional environments, such as you identified in step 2. For each well mark some/all of the parasequence sets (long arrows) and parasequence set boundaries (squiggly lines). |
| 4 | There are two (2) parasequence set boundaries on the cross section. Did you find both? Remember that parasequence set boundaries can be surfaces of erosion, so parasequences below these boundaries can be partially removed (i.e., cut into) and parasequences above these boundaries can be areally limited (i.e., localized fill within the erosional cuts). Correlate the parasequence set boundaries from one well to the next. |

Continued on next page

Exercise 20a: Parasequence Correlation   continued

| Step | Action |
| --- | --- |
| 5 | With a framework of parasequence set boundaries in place, correlate the parasequences from one well to the next. As you correlate the parasequence boundaries, keep in mind the thickness changes of the parasequences as they extend across the log section. Do the thickness changes seem geologically reasonable? There can be thickness changes, but they should not be drastic. |
| 6 | Finally, you can interpret the distribution of the depositional environments between the wells. Draw facies boundaries, i.e., the transition from fluvial to shallow marine sandstones. Now you can color-code the four (4) depositional environments and shade them in across the entire cross section. |