Exercise 20b: Mapping the Top and Base of Reservoir

| Objective | In this exercise, you will use the seismic lines from Exercise 19 to complete an interpretation of the top and base of the main reservoir zone in the Barracouta area. You will generate two (2) time-structure maps – for the top and base of this reservoir interval. This will define the reservoir interval from which we can determine other reservoir parameters, such as time thickness. |
| --- | --- |

| Materials | - Exercise instructions - 3D seismic displays from Exercise 19 - Four (4) clean copies of the basemap - Color pencils |
| --- | --- |

| Introduction | In Exercise 19, you started a structural interpretation of the 3D seismic data. Now you are to map the top and base of the main reservoir. For now, we will use the top of Latrobe (TOL) horizon as the top of the reservoir. The base reservoir horizon has been marked on the several of the 3D lines given to you in Exercise 19. The top and base horizons have also been marked on the 1100 ms time slice. Your task is to complete the mapping of both horizons and then generate two (2) time structure maps. These horizons will be used in Exercise 22 to get the thickness of the reservoir (in TWT thickness). There are two (2) ways to map the two horizons (top and base). One is to interpret the top on all lines and then go back and interpret the base. The other is to map both horizons simultaneously. Since it takes time to find two (2) intersecting lines and ‘tie’ them correctly, it is more efficient to map the two (2) horizons simultaneously – but do the mapping however you feel more comfortable. |
| --- | --- |

Continued on next page

Exercise 20b: Mapping the Top and Base of Reservoir   continued

| Step | Action |
| --- | --- |
| 1 | You have two (2) copies of the basemap for mapping interpretation extent, one marked “Top of Reservoir” and the other marked “Base of Reservoir.” Take inventory of where you have been given the two (2) horizons (i.e., use a color pencil to mark on the first basemap the lines where the TOL horizon has been interpreted. Use a different color pencil to mark on the second basemap the lines where the base reservoir horizon has been interpreted. |
| 2 | The two (2) horizons have been marked on the 1100 ms time slice. Try tying a crossline (e.g., Xline 3400) to this time slice and transferring the two (2) horizons from the interpreted time slice to the uninterpreted crossline. After you tied a crossline to the time slice, try tying the two (2) horizons from the interpreted 1100 ms time slice to an uninterpreted inline. |
| 3 | Complete the interpretation of the two (2) horizons. In the process, try tying an interpreted inline or crossline to an uninterpreted time slice. Do your interpreted horizons make sense on the time slice view? If the horizons appear to be offset by faults, add the faults to your inlines and crosslines. There probably will NOT be time for you to develop a set of 3D consistent fault planes, but you now know the process (from Exercise 19). |

Continued on next page

Exercise 20b: Mapping the Top and Base of Reservoir   continued

| Step | Action |
| --- | --- |
| 4 | Develop a strategy to map the two (2) horizons and divide the work among team members, if you are working with a partner. After the two (2) horizons are marked on enough lines/slices, you can start measuring the TWT to the TOL horizon. This task is easier when working in a team of two (2) so that one person calls out the TWT values and the other person posts (writes) the values on the basemap. To read the TWT values, use the time scale at either end (left or right) of any inline or crossline as a “marked ruler.” NOTE: Since the horizons tie, the TWT at any intersection will be the same on the intersecting inline and crossline. Thus, you can read/post TWT values either on the inlines OR the crosslines – not both. |
| 5 | Now contour the TWT values for the TOL (top of reservoir) horizon. This gives a graphic representation for the 3D surface for the top of reservoir. Start contouring values of 1000 ms and less (the crest of the anticline). Then, contour the values below 1000 ms; a contour interval of 100 ms is adequate. |
| 6 | Repeat steps 4 and 5 for the base of reservoir horizon. Your final products for Exercise 20 is two (2) time structure maps: - Top of Reservoir (TOL). - Base of Reservoir. |