Exercise 24: Seismic Attribute Analysis

| Objective | In this exercise, you will use two (2) seismic attribute maps to identify areas where good-quality seal rocks overlay good-quality reservoir rocks. |
| --- | --- |

| Materials | - Exercise instructions - Two (2) seismic attribute maps - Orange time structure map - Tracing paper |
| --- | --- |

| Introduction | Seismic data in this area has been converted to quadrature phase. This means that the interface between two units occurs at the zero crossing. The loop (peak or trough) above the zero crossing is affected by the rock properties in the overlying sequence. The loop below the zero crossing is affected by the rock properties in the underlying sequence. For our zone of interest there is a good correlation between reservoir quality and the trough below the orange horizon. Similarly there is a good correlation between seal quality and the peak above the orange horizon. By extracting amplitude values for the trough below and the peak above the orange horizon, we can qualitatively predict where a good reservoir rock is capped by a good sealing facies. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Place the tracing paper on the “Above-Orange Peak Amplitude" map and register the corners. |

Continued on next page

Exercise 24: Seismic Attribute Analysis   		continued

| Step | Action |
| --- | --- |
| 2 | Draw polygons around areas with moderate- to strong-positive amplitude responses (yellow-green-blue). Inside these polygons, draw a few diagonal lines (e.g., / / / / ). |
| 3 | Register the tracing paper on the “Below-Orange Peak Amplitude" map. Draw polygons around areas with moderate- to strong-negative amplitude responses (green-yellow). Inside these polygons draw a few diagonal lines with a different orientation (e.g., \ \ \ \ ). |
| 4 | Look for areas where the response both above and below the orange horizon has moderate- to strong-amplitude responses. These will be on your tracing paper where the two different patterns overlap. Draw polygons encloses the coincidence of both patterns and shade these areas. These areas, at least based on our simple model, will have a good-quality reservoir rock capped by a good-quality sealing facies. |
| 5 | Register the tracing paper on the “Orange Time Structure" map. Do any of your shaded areas lie on top of a structural high? These would be potential drilling targets. |