Exercise 25: Reservoir Quality

| Objective | Without computers, it is not practical to have you generate a reservoir quality map. Instead you will analyze the crossline that goes through the Barracouda-1 well location. Therefore, you will work in terms of cross-sectional area rather than 3D volume. A map of reservoir quality is important for several reasons. It is vital input to an estimate of in-place and recoverable HC volumes (HC reserves). It will also dominate our initial field development plan. |
| --- | --- |

| Materials | - Exercise instructions - Crossline 3445, which crosses the Barracouta-1 well location - A computation table - Color pencils and erasers - A scale in units of TWT milliseconds (bottom of page 3) |
| --- | --- |

| Introduction | Lecture 25 presented the primary factors that control reservoir quality. You will complete a computation table step-by-step to get an estimate of the in-place HCs just for the Crossline that intersects Barracouta-1. Computer tools available in interpretation software allow you to walk through the same basic procedure to obtain an estimate of in-place HC volume for the field or prospect that you are charged to work. For this exercise, you will have to fill in only columns I, J, and K on the computation sheet, with a few exceptions. |
| --- | --- |

Continued on next page

Exercise 25: Reservoir Quality   				continued

| Step | Action |
| --- | --- |
| 1 | Examine Crossline 3445; this line goes through the Barracouta-1 well location. From the well, we know that the gas-water contact (GWC) is at 976 ms. Draw a horizontal line on the seismic line at this two-way time value. In this basin, there are significant lateral velocity changes (large channels in which the sedimentary fill has slower velocities than the rocks they replaced). As a result, the GWC is not flat in TWT. You will map the GWC in another exercise. For now, we will assume that the GWC is flat (i.e., horizontal) on the seismic line. |
| 2 | Note the vertical columns on the seismic line that are labeled “A” through “T.” Each column represents ½ kilometer in width along the seismic line. Note that the first row on the computation sheet is cumulative reservoir width. Columns A through E have zeros since these are downdip of where the GWC intersects the horizon at the top of the reservoir. Complete row 1 by incrementing each column to the right of E by ½ km until you get to the east edge of the field (column S) where the GWC again intersects the top reservoir horizon. |
| 3 | Note row 2 on the computation sheet. This is for the reservoir thickness in TWT (in unit of ms) for each column above the GWC. The first few values are provided. Complete row 2. Use the scale at the bottom of the computation sheet to measure time thickness on Figure 1. |

Continued on next page

Exercise 25: Reservoir Quality   				continued

| Step | Action |
| --- | --- |
| 4 | Next you will convert the TWT thickness to a thickness in meters. From the well logs we know that the average velocity for the reservoir interval is 2500 m/s. The thickness in meters is (TWT/2000) * 2500. Use this equation and the value in row 2 to complete row 3 (thickness in meters). |
| 5 | For simplicity, we will consider the reservoir to consist of three reservoir intervals (upper, middle, lower) and assign EODs to each. The EODs are indicated on the calculation sheet in rows 4, 5 and 6 (F = fluvial; DP = delta plain; DF = delta front). Figure 2 shows the distribution of these facies on a datumed (flattened) version of the seismic line. |
| 6 | We will use the EODs to assign both the net-to-gross (N:G) and the porosity (Φ) of each column. For the fluvial environment, we will use a N:G of 50% and a Φ of 16%. For the delta plain environment, we will use a N:G of 65% and a Φ of 18%. For the delta front environment, we will use a N:G of 80% and a Φ of 20%. Complete rows 7, 8, 9, 10, 11 and 12. |
| 7 | We will assume that the three intervals are 1/3 the total thickness at each location (see the datumed seismic line, Figure 2). This makes it easy to obtain the average N:G and porosity of each column. Complete rows 13 and 14. |
| 8 | The width of each column is 0.5 km. The height in meters is in row 3. To get the reservoir cross-sectional area, multiply 0.5 km by the thickness in meters by (.001 km/m). This will give the area in square km. Complete rows 15. |

Continued on next page

Exercise 25: Reservoir Quality   				continued

| Step | Action |
| --- | --- |
| 9 | The reservoir rock area is the total area multiplied by the N:G. Multiply row 15 by row 13; place the product in row 16. |
| 10 | The reservoir pore area is the reservoir area multiplied by the porosity. Multiply row 16 by row 14; place the product in row 17. |
| 11 | Finally, the HC pore area is the reservoir pore area multiplied by HC saturation. We will use 90% for the HC saturation. Multiply row 17 by 0.9; place the product in row 18. |
| 12 | Sum all the values in row 18 to get the total HC area in units of square km. |
| 13 | If the HC area you just calculated was the average for a field that is 5 km long, what would the HC volume be in cubic km? (Volume in km3 = HC Area * 5 km) If the reservoir contained oil, the conversion from km3 to barrels is 6290 MBO/km3. (HC volume in million barrels = 6290 * Volume in km3). |
| 14 | One more assumption, if the recovery efficiency is 25%, how much oil could be produced from this size reservoir? (Produced volume = 0.25 * in-place volume). |