Exercise 26b: DHI Modeling - North Sea

| Objective | You will consider the reflection coefficients and seismic responses of a reservoir that has a gas cap and an oil leg and that is capped by shale. You will use physical properties that are representative of the North Sea. First, you calculate the RCs. Then, you will draw approximate wavelets on a simple cross. |
| --- | --- |

| Materials | - Exercise instructions - Pencil - Calculator |
| --- | --- |

| Introduction | In step 1, you will be given some velocity and density information. In steps 2 and 3, you will calculate five (5) RCs: 1) Shale over water sand 2) Shale over oil sand 3) Shale over gas sand 4) Gas sand over oil sand - a GOC (gas-oil contact) 5) Oil sand over water sand - an OWC (oil-water contact) Then, given the zero-phase response for the shale over water sand case (case #1), you will draw some additional simple zero-phase wavelets on a simple cross section. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Below are some velocities and densities that are representative of the North Sea for four (4) lithology-fluid pairs at our depth of interest: 1) Shale: V = 8000 ft/s; Density = 2.20 gm/cc 2) Water sand: V = 8700 ft/s; Density = 2.32 gm/cc 3) Oil sand: V = 8300 ft/s; Density = 2.29 gm/cc 4) Gas sand: V = 7700 ft/s; Density = 2.22 gm/cc. |

Continued on next page

Exercise 26b: DHI Modeling - North Sea 		continued

| Step | Action |
| --- | --- |
| 2 | Calculate the impedance for all four lithology-fluid pairs: 1) Shale: I = ____________ 2) Water sand: I = ____________ 3) Oil sand: I = ____________ 4) Gas sand: I = ____________ |
| 3 | Calculate five (5) reflection coefficients, paying attention to which are positive and which are negative. Recall that: Ref. Coeff = Ibelow – Iabove Ibelow + Iabove 1) Shale over water sand; RC = ________ 2) Shale over oil sand; RC = ___________ 3) Shale over gas sand; RC = __________ 4) Gas sand over oil sand; RC = ________ GOC 5) Oil sand over water sand; RC = _______ OWC |
| 4 | Figure 1 shows a simple cross section of a reservoir sand capped by a shale. There are four (4) places for you to draw simple wavelets. At location A, a zero-phase wavelet has already been drawn. Draw wavelets at locations B, C, D, and E. Keep in mind the sign and the magnitude of the RC at A. The other wavelets will have the same polarity only if the RC has the same sign (+ or -) as at A. Approximate the wavelet amplitude based on the magnitude of the other RCs relative to the RC amplitude at A. |

Continued on next page

Exercise 26b: DHI Modeling - North Sea 		continued

Figure 1.   Cross section for a simple reservoir model. A wavelet has been drawn at Location A.