Exercise 26c: DHI Mapping

| Objective | The gas-water contact (GWC) at Barracouta can be observed on the seismic lines. Although the fluid contact is flat in depth, lateral velocity changes above the reservoir makes the GWC non-horizontal on the seismic lines. Given some starting interpretation, you will complete the mapping of the GWC. A map of the limits of the GWC allows us to get the area for the field with a high level of confidence. It also allows us to get gross reservoir thickness with confidence. We will use the GWC to obtain an estimate of the EUR (estimated ultimate recovery) in Exercise 29. |
| --- | --- |

| Materials | - Exercise instructions - The seismic inline through the well with the GWC interpreted - 6 seismic crosslines with the GWC interpreted - 7 seismic inlines without the GWC interpreted - A basemap - Pencils and erasers |
| --- | --- |

| Introduction | You will begin by looking at the criteria used to map the GWC on several lines. Then you can tie the crosslines to the inlines and transfer the GWC surface (horizon). This will allow you the complete the GWC interpretation quickly. Once you have interpreted the GWC, you will post where the GWC intersects the TOL (top of reservoir) horizon on the basemap. These intersection points delimit the northeast portion of the field. We have a high degree of confidence in the field area after we conduct this type of mapping over the entire field. We can also get a fairly accurate estimate of gross reservoir thickness. You will not do this last step simply due to time constraints. |
| --- | --- |

Continued on next page

Exercise 26c: DHI Mapping   				continued

| Step | Action |
| --- | --- |
| 1 | Examine inline 1915, which goes through the Barracouta-1 well location. The GWC, obtained from the well logs, has been interpreted. Note the reflection geometries on the north flank, especially how they are disrupted at the GWC. There is some evidence for the GWC on the south flank. Across the crest of the structure, where the GWC does not cut across the stratigraphy (reflections), there is not much evidence for the contact. The evidence on each flank allows us to project the GWC through the center of the field. |
| 2 | Start to transfer the GWC that is interpreted on the crosslines to the uninterpreted inlines. I recommend you start with inline 2100 and move systematically to the west. Transfer the interpretation at each tie point on an inline, and then draw in the GWC. Remember that the best evidence is on the flanks, where the fluid contact cuts across the rock layering (reflections). There is not much evidence for the fluid contact in the center of the field where the contact parallels the seismic reflections. |
| 3 | After you have interpreted the inlines, you are ready to map the extent of the gas. Use a colored pencil to mark where on each inline and crossline the gas is present. You do not have to be very precise (e.g., on inline 1915 the gas extends from about ½ way between 3000 and 3100 on the south to about ½ way between 3700 and 3800 on the north.) |

Continued on next page

Exercise 26c: DHI Mapping   				continued

| Step | Action |
| --- | --- |
| 4 | Once you have mapped the extent of the gas on each inline and crossline, draw a polygon to represent the extent of the field at its northeastern end. |