Exercise 27: AVA Analysis

| Objective | You will use some information from a seismic gather to predict hydrocarbon fill levels (gas cap, oil leg, water leg) within a reservoir on a seismic line. |
| --- | --- |

| Materials | - Exercise instructions - Pencils - Calculator |
| --- | --- |

| Introduction | Figure 1 shows a seismic profile across an anticlinal trap. There is evidence for at least one (1) fluid contact. If there is only one fluid contact, than it could be gas on water, oil on water, or gas on oil. Alternatively, this could be a false fluid contact (e.g., multiple, stratigraphic surface). Can we use AVO or AVA data to be more quantitative in predicting the fluids in the target interval? |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Figure 2 shows a gather from the shot point indicated in Figure 1. An arrow marks the target reservoir. Note that, on the gather, the amplitude of the trough increases significantly with angle. What class AVA anomaly is this? Class _______ |
| 2 | Table 1 on page 3 lists nine (9) angle-amplitude pairs from this gather. Use the graph on page 3 to plot these nine (9) data pairs. |

Continued on next page

Exercise 27: AVA Analysis   				continued

Figure 1.  A seismic line across an anticlinal trap.  There is evidence for at least one fluid contact. Our goal is to predict the fluids in the target reservoir.

Figure 2.  A seismic gather from the seismic line shown in Figure 1.

Continued on next page

Exercise 27: AVA Analysis   				continued

0       4       8     12     16    20     24    28     32    36

-20

-40

-60

-80

-100

-120

0

-140

Continued on next page

Exercise 27:  AVA Analysis   				continued

| Step | Action |
| --- | --- |
| 3 | Draw a straight line fit to the nine (9) data points you just plotted on page 3. What is the amplitude at 0° based on your line? __________ What is the slope of your line? _____________ |
| 4 | From elsewhere in this basin, we obtained representative velocities and densities for shales, brine sands, oil sands and gas sands from depths comparable to our target. We modeled the AVA response for shale on top of the three (3) types of fluid-filled sands, and plotted the modeling results on the intercept-slope graph in Figure 3. Note in Figure 3 the separation of the shale over sand with the three (3) different fluids. Now use the intercept and slope you calculated in Step 3 to plot on Figure 3 the AVA response for the gather shown in Figure 2. Based on the modeling results, what fluid-filled sand do you expect at the shot point associated with the gather in Figure 2? |

Continued on next page

Exercise 27: AVA Analysis   				continued

Figure 3. An AVA intercept versus slope plot that shows the results of our AVA modeling efforts of well data. Shale over the three (3) types of fluid-filled sands occupies different regions. This indicates that AVA predictions are feasible for this area of interest.

| Step | Action |
| --- | --- |
| 5 | Figure 4 shows the AVA attributes for the reflection at the top of the potential reservoir over the anticline (more than just on the seismic gather shown in Figure 1). The distribution of points looks similar to the modeling results (Figure 3). Thus the real seismic data points were color-coded based on the predicted fluids in the target reservoir (gas, oil, brine). |

Continued on next page

Exercise 27: AVA Analysis   				continued

Figure 4. An AVA intercept versus slope plot showing the results of an AVA analysis for the reflection at the top of the target reservoir near the crest of the anticline. The data points have been color-coded based on the predicted fluid in the sands.

| Step | Action |
| --- | --- |
| 5 Cont. | Software tools were used to reference the color-coded AVA attributes along the top reservoir horizon on the seismic line (Figure 5). Use Figure 5 to predict the hydrocarbon distribution at the crest of the anticline. Draw in any inferred fluid contacts. Has the AVA analysis reduced the risk for this lead? Explain. |

Continued on next page

Exercise 27: AVA Analysis   				continued

Figure 5.  The predicted fluids based AVA intercept and slope and their distribution shown in Figure 4 has been displayed on the seismic cross section. Use this figure to predict HC distribution in the reservoir.