Exercise 28: Time-Depth Conversion

| Objective | In this exercise, you will convert several horizons marked on a TWT seismic section to horizons in depth (meters). You will compare the highest point in TWT to the highest point on the depth section. |
| --- | --- |

| Materials | - Exercise instructions - 1 seismic line in TWT - A graph to sketch six (6) horizons in depth (meters) |
| --- | --- |

| Introduction | Figure 1 shows a seismic traverse across the crest of the Barracouta structure. Nine (9) horizons have been mapped; the magenta horizon is the Top of Latrobe (TOL) unconformity, which is very close to the top of the main reservoir interval. The velocities above the reservoir are not simple; there are significant lateral velocity variations. As a result, the horizon configuration in time is not the same as the true horizon configuration in depth. The velocity variations are constant or slowing varying within the five (5) stratigraphic intervals defined by the horizons above the TOL. Given a horizon-keyed velocity model, you will calculate the depth for six (6) horizons (from water bottom to the TOL) and then sketch a cross-section that has the vertical scale in meters. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Examine Figure 1 and the six (6) horizons from the water bottom to the TOL horizon (magenta). The TWT values at each of the 10 locations marked A through J are given in Table 1. At which of the ten (10) locations is the crest of the TOL horizon in time, i.e., which location has the smallest TWT value for the magenta horizon (use Table 1 to answer)? Crest in TWT at location __________ |

Continued on next page

Exercise 28: Time-Depth Conversion   		continued

Figure 1. Seismic traverse across the crest of the Barracouta structure. The six (6) horizons from blue to magenta define five (5) stratigraphic intervals. Velocities are constant or slowly varying within each of these intervals (sequences).

TABLE 1

Horizons in Units of Two-Way Time

| TWT | A | B | C | D | E | F | G | H | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WB | 0.060 | 0.075 | 0.075 | 0.080 | 0.100 | 0.140 | 0.170 | 0.175 | 0.170 | 0.175 |
| Cyan | 0.180 | 0.210 | 0.230 | 0.240 | 0.285 | 0.360 | 0.440 | 0.540 | 0.590 | 0.635 |
| Yellow | 0.300 | 0.320 | 0.350 | 0.355 | 0.400 | 0.470 | 0.525 | 0.720 | 0.800 | 0.890 |
| Purple | 0.430 | 0.460 | 0.465 | 0.530 | 0.540 | 0.690 | 0.900 | 1.040 | 1.125 | 1.160 |
| Orange | 0.615 | 0.665 | 0.690 | 0.780 | 0.790 | 0.910 | 1.090 | 1.250 | 1.320 | 1.320 |
| Magenta | 1.020 | 1.030 | 1.015 | 0.940 | 0.950 | 1.025 | 1.200 | 1.475 | 1.620 | 1.675 |

Continued on next page

Exercise 28: Time-Depth Conversion   		continued

| Step | Action |
| --- | --- |
| 2 | Given, in Table 1, the TWT value for each horizon, we can get the TWT thickness for each of the five (5) intervals at the 10 locations. Most of the TWT thickness values are provided in Table 2. Complete Table 2 by filling in values at locations D and E. |

TABLE 2

Time Thickness for the Five Intervals (Sequences)

| Time Thickness | A | B | C | D | E | F | G | H | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water Layer | 0.060 | 0.075 | 0.075 | 0.080 | 0.100 | 0.140 | 0.170 | 0.175 | 0.170 | 0.175 |
| Pleisto | 0.120 | 0.135 | 0.155 |  |  | 0.220 | 0.270 | 0.365 | 0.420 | 0.460 |
| U Plio | 0.120 | 0.110 | 0.120 |  |  | 0.110 | 0.085 | 0.180 | 0.210 | 0.255 |
| L Plio | 0.130 | 0.140 | 0.115 |  |  | 0.220 | 0.375 | 0.320 | 0.325 | 0.270 |
| Mio | 0.185 | 0.205 | 0.225 |  |  | 0.220 | 0.190 | 0.210 | 0.195 | 0.160 |
| Lake Entrance | 0.405 | 0.365 | 0.325 |  |  | 0.115 | 0.110 | 0.225 | 0.300 | 0.355 |

| Velocities | If we used a simple, constant velocity model that was a single function of depth, then the overall configuration of the horizons would be the same in time and depth – it would just be a simple re-scaling (stretching - squeezing) of the values. The crest of the anticline would be at the same location in depth as it is in time. This was shown in the lecture slides. However, the velocity structure in this area is not so simple. The velocities are primarily related to the five stratigraphic intervals. Detailed velocity analyses were done every 2 kilometers. This indicated that the velocities for the water layer and the Pleistocene are nearly constant. We can use velocities of 1500 m/s and 1540 m/s for these two intervals (see Table 3). |
| --- | --- |

Continued on next page

Exercise 28: Time-Depth Conversion   		continued

| Velocities Cont. | For the Upper and Lower Pliocene, velocities increase from updip to downdip (Table 3). This is due to an increase in average depth and the state of sediment compaction. The Miocene interval has a mixture of clastics and carbonates (carbonates have much faster velocities). Carbonate percentage is high updip and decreases downdip. As a result, velocities vary between 4450 m/s updip to 3550 m/s downdip (Table 3). The Lakes Entrance formation has silts and shales updip; it is a pure deep marine shale downdip. Its velocity varies from 1775 m/s updip to 1525 m/s downdip (Table 3). |
| --- | --- |

| Step | Action |
| --- | --- |
| 3 | Given the time thicknesses in Table 2 and the velocities in Table 3, we can calculate the thickness of each sequence in units of meters (Thicknessmeters = ThicknessTWT * Velocity / 2). Table 4 has most of the interval thicknesses filled in. Complete Table 4 by adding values for locations D and E. |

TABLE 3

Interval Velocities by Sequence

Continued on next page

Exercise 28: Time-Depth Conversion   		continued

TABLE 4

Thickness of Each Sequence in Meters

| Step | Action |
| --- | --- |
| 4 | You can now build a table for the depth (in meters) for each horizon. For example, the depth of the top of the Pleistocene equals the water layer thickness; the depth to the top of the Upper Pliocene equals depth to the top of the Pleistocene plus the thickness of the Pleistocene. Complete Table 5. |

TABLE 5

Depths of Each Horizon in Meters

Continued on next page

Exercise 28: Time-Depth Conversion   continued

| Step | Action |
| --- | --- |
| 5 | Use Figure 2 to sketch the depths of the horizons in Table 5. This is the equivalent depth section for the seismic traverse, based on the detailed velocity analyses. Compare the overall shape of the horizons on the seismic line (Figure 1) and the depth section (Figure 2). At which of the 10 locations is the crest of the TOL horizon in depth (magenta horizon) (use Table 5)? Crest in TWT at location __________ |

Continued on next page

Exercise 28: Time-Depth Conversion   		continued