Exercise 29: Using Inversion Data

| Objective | In this exercise, you will consider how an absolute impedance trace is constructed and used to evaluate a reservoir interval. |
| --- | --- |

| Materials | - Exercise instructions - Pencil |
| --- | --- |

| Introduction | As explained in the lecture, zero-phase seismic data allows us to identify major impedance boundaries. Once an important interval is identified and mapped (e.g., a reservoir zone), interpreters often transform the data into either relative or absolute impedance so that seismic attributes can be used to quantify interval properties. The example you will work is from Western Canada. A Triassic formation is composed of thick sandstone bodies that are encased in shales. Many of these sands have been found to be economic hydrocarbon reservoirs. The sands have higher impedance than the shales. In addition, there is a good correlation between higher impedance values and higher reservoir quality (a combination of greater sand thickness, higher net-to-gross and higher porosity). Before this exercise, our team used a zero phase seismic data volume to map the top of the Triassic formation of interest. Then, the team transformed the volume into a relative impedance 3D volume. At the start of this exercise, you will examine the relative impedance trace that is at the location of a well. Then, you will do a few calculations to complete an absolute impedance trace. This will allow you to compare the usefulness of the two (2) types of impedance data. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Figure 1 shows the impedance curve from a well (from the sonic and density logs) and the corresponding relative impedance trace. Note how the relative impedance trace lacks the low frequency trends that are seen in the impedance curve. |

Continued on next page

Exercise 29: Using Inversion Data   			continued

Figure 1. The impedance curve from a well (left) and the relative impedance trace at the well location (right). Note that the relative impedance trace lacks the low frequency trends seen in the impedance curve.

Continued on next page

Exercise 29: Using Inversion Data   			continued

| Step | Action |
| --- | --- |
| 2 | Using only the relative impedance trace, can you identify a high impedance zone that might indicate the presence of a quality reservoir? In other words, if you did not have the well data at this location, would you predict that there is a reservoir worth drilling a well to discover? |
| 3 | We want to get an impedance response that looks more like the log response. If you have not noticed, our zone of interest in around 1030 to 1040 ms. Seismic data typically has a frequency range of 10 to 50 Hz. We do not have methods to expand the frequencies greater than 50 Hz. However, we can estimate and add in the low frequency (less than 10 Hz) response. Where we have well data, we can filter the log data so that we can capture the low frequency component. Away from wells, we can use the stacking velocities that our seismic data processors derive from gather analysis. Of course, the low frequency component derive from gathers is not as precise as filtered log data. Note in Figure 2 the low frequency trend that was obtained by filtering the log data. |

Continued on next page

Exercise 29: Using Inversion Data   			continued

Figure 2.  The impedance curve (from Figure 1) with the low frequency component (dotted curve) superimposed. The low frequency component was obtained by filtering the impedance curve for frequencies in the 0 to 10 Hz range.

Continued on next page

Exercise 29: Using Inversion Data   			continued

| Step | Action |
| --- | --- |
| 4 | Figure 3 shows the relative impedance curve and the low frequency component derived from filtering the impedance curve for the interval from 920 to 1040 ms. If we add these two curves together, we get what is referred to as the absolute impedance. The frequency range for this curve will be from close to zero out to about 50 Hz. We still do not have the higher frequencies, but the absolute impedance curve looks much more like the log curve. Table I has the data values for the relative impedance curve and the low frequency component. For each row, add the numbers in columns 2 and 3 and place the sum in column 4. Column 4 is our estimate of the absolute impedance. |

Figure 3.  A display of the relative impedance curve (left) and the low frequency component (right) for our zone of interest. Absolute impedance is the sum of these two curves.

Continued on next page

Exercise 29: Using Inversion Data   			continued

Table I

Values of relative impedance (column 2) and the low frequency component (column 3) as a function of TWT

| TWT | Relative Impedance | Low Frequency | Absolute Impedance |
| --- | --- | --- | --- |
| 980 | 18 | 75 | 93 |
| 984 | 25 | 70 | 95 |
| 988 | 0 | 65 | 65 |
| 992 | 8 | 60 | 68 |
| 996 | 8 | 55 | 63 |
| 1000 | -2 | 50 | 48 |
| 1004 | -45 | 45 |  |
| 1008 | -40 | 40 |  |
| 1012 | -34 | 34 |  |
| 1016 | -28 | 28 |  |
| 1020 | -23 | 23 |  |
| 1024 | -18 | 18 |  |
| 1028 | -13 | 13 |  |
| 1032 | -10 | 10 |  |
| 1036 | 12 | 6 | 18 |
| 1040 | -5 | 2 | -3 |

| Step | Action |
| --- | --- |
| 5 | Plot your values from column 4 on the graph in Figure 4. The absolute impedance values for the upper part of the TWT range in Figure 4 have been plotted to save you time. |
| 6 | Consider how the identification of a higher impedance sand unit is easier to make with more confidence in Figure 4 than in the left side of Figure 3. |

Continued on next page

Exercise 29: Using Inversion Data   			continued

Figure 4.  A display of the absolute impedance curve. Values above 980 ms have been provided. Complete the curve by plotting your sums from column 4 and sketching in the lower portion of the curve.