Exercise 30a: Estimating Ultimate Recovery

| Objective | We will now provide you with enough information on the Barracouta discovery so that you can estimate the recoverable reserves, i.e., the estimated ultimate recover (EUR) of this undeveloped field. The gas EUR will assign a value to the field (assuming some future price for gas). This value will be compared to the costs to first develop and then operate the field for a certain number of years. That value minus the costs gives us the profit. If the estimated profit is acceptable, then we can propose proceeding to the field development phase. |
| --- | --- |

| Materials | - Exercise instructions - Computation sheet, with figures - Pencils and erasers |
| --- | --- |

| Introduction | You will use a set of numbers to obtain several estimates of the ultimate gas recovery for Barracouta. These numbers come primarily from seismic data analysis. You will make three (3) estimates: - A “most likely” case - A minimum (low-side) case – could be as small as X. - A maximum (high-side) case – could be as large as Y. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | There is evidence for a fluid contact on the seismic. Assuming that the mapping of the DHI was done properly, we get an area of 90 km2 (Figure 1). Given data resolution and data quality issues, we might make the area a little smaller (low-side case) (Figure 2a) or a little larger (high-side case) (Figure 2b). These areas have been entered into row one of the computation sheet. |

Continued on next page

Exercise 30a: Estimating Ultimate Recovery   cont.

| Step | Action |
| --- | --- |
| 2 | In part of the seismic data analysis, the reservoir interval was subdivided into three depositional environments: fluvial, delta plain, and delta front. We will assume different averages for net-to-gross and porosity for each depositional environment. Further analysis led to a subdivision of the reservoir interval into three (3) units: upper, middle, and lower. Figure 3 shows an interpretation for how the three (3) depositional environments stack in the three (3) intervals of the reservoir. Figure 4 displays the product of net-to-gross (N:G) multiplied by the porosity. For example, the pink region on the west is where all three units are fluvial. For the fluvial rocks, the N:G is 0.50 and the porosity is 0.16, so the product is 0.08. Given Figure 4, make three (3) estimates for the average of N:G times porosity for the entire field: A most-likely field-wide average, A reasonable minimum average and A reasonable maximum average. Enter your estimates on line 2 of the computation sheet. |
| 3 | With the top of the reservoir mapped (the TOL horizon) and the GWC mapped, we can get the time-thickness (in ms) of the gross reservoir where it contains gas. This is shown in Figure 5. We assumed an interval velocity of 2500 m/s to get a gross reservoir in meters (Figure 6). Given Figure 6, make three (3) estimates for a field-wide average for reservoir thickness. We want to work in units of kilometers, so divide your estimates of thickness in meters by 1000. Enter your three (3) estimates (in kilometers) on line 3 of the computation sheet. |

Continued on next page

Exercise 30a: Estimating Ultimate Recovery   cont.

| Step | Action |
| --- | --- |
| 4 | Now you can calculate the HC pore volume at reservoir conditions in units of cubic kilometers. Multiply rows 1, 2, 3 and 4 for the three different estimates and place the products on line 5 of the calculation sheet. |
| 5 | The recovery efficiency for this reservoir was thought to be high. For our estimates, use the recovery efficiencies already entered on line 6 of the computation sheet. |
| 6 | Next, multiply line 5 by line 6. The product is the recoverable gas volume in cubic kilometers at reservoir conditions (i.e., reservoir pressure and temperature). Enter this on line 7. |
| 7 | Since one cubic kilometer equals one billion cubic meters, line 5 is also the value of reservoir gas in Bcm (billion cubic meters). As the gas rises to the surface as it is produced, it expands as both the temperature and pressure drops. Multiply line 7 by 111 to convert from gas volume at reservoir conditions to gas volume at the surface. Enter the value on line 8. |
| 8 | For those accustomed to English units of volume, you have one more step. Multiply the number on line 8 by 35.3 and write the product on line 9. This is the surface gas volume in billion cubic feet (BCF). |