Exercise 30b: Selecting Development Well Locations

| Objective | You will use the information you have on hand to propose two (2) development-stage wells. Your team will consider where to place these wells and be ready to explain the reasons for your proposed locations. |
| --- | --- |

| Materials | - Exercise instructions - Reservoir time thickness map |
| --- | --- |

| Introduction | Management is interested in developing the Barracouta Field. However, before agreeing to spend $500 million on a platform, they want to drill some development-stage wells. They have budgeted a total of two (2) wells. Given the data and analyses that you have, your team needs to decide where to locate these wells. You want to maximize the information that you will obtain from these two (2) new wells. As a team, consider what some of the biggest uncertainties are. Where can you drill two (2) wells to reduce these uncertainties? Be ready to give a short explanation of where you would place the wells and why. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | You have detailed information at the Barracouta-1 well location. You have good seismic coverage for the rest of the field. As a team, consider what is still unknown or highly uncertain about the newly discovered Barracouta Field. Can some of these unknowns or uncertainties be eliminated or significantly reduced by drilling a well in a given location? Compile a list of 3 or 4 things you would like to determine before your company invests in a costly platform. |

Continued on next page

Exercise 30b: Development Well Locations    continued

| Step | Action |
| --- | --- |
| 2 | Now that your team has some technical objectives (from step 1), consider where you should drill to evaluate these technical objects. For example, had the GWC (gas-water contact) not been intersected by the discovery well, a worthwhile objective would have been to drill further down dip to verify the depth of the GWC. Without this depth, there would be a lot of uncertainty as to how much gas was in the structure. |
| 3 | The map of the thickness of the reservoir above the GWC that was shown in Exercise 30a (Figure 6) is very useful. It is repeated as Figure 1 in this exercise. |
| 4 | Several teams will be asked to give a brief (< 5 minutes) synopsis of where they would drill and what information they hope to obtain. Prepare to give such a report. |