Exercise 32a: Exploitation-Scale Mapping

| Objective | In this exercise, you will use lines out of the 3D survey to start mapping the stratigraphy at the much finer scale that is needed for planning field development and initial field production. This will build on the marker horizons picked on well logs and tied to the seismic data in Exercise 31. |
| --- | --- |

| Materials | - Exercise instructions - Three (3) base maps - Eighteen (18) seismic lines - Colored pencils, erasers |
| --- | --- |

| Introduction | You have tied four (4) wells to the 3D seismic data and worked six (6) marker horizons in Exercise 31. You correlated these marker horizons between the well locations using a seismic traverse through the well locations. The next step is to start to map more detailed horizons in the Upper Latrobe, our primary reservoir unit. You are given eighteen (18) seismic lines; eight (8) of these lines have been interpreted for you. Your task is to start mapping these horizons on the ten (10) uninterpreted lines taken out of the 3D survey. The objective is NOT to complete every horizon on every line; rather, the objective is to appreciate the level of stratigraphic detail that can be extracted from the seismic data given some well data. You will map SEVEN seismic horizons – the six (6) markers that you worked with in Exercise 31 plus a horizon colored green on the interpreted lines. This horizon was added to help work out the missing sediments due to erosion associated with the Top of Latrobe (TOL) erosional unconformity. |
| --- | --- |

Continued on next page

Exercise 32a:  Exploitation-Scale Mapping   continued

| Step | Action |
| --- | --- |
| 1 | Your first step is to take inventory of what you have. You will find that eight (8) of the eighteen (18) lines have been interpreted. The interpreted lines are the heavier red lines on the base maps. You have three (3) copies of the base map. I recommend that you use the first map to mark the lines you work as you proceed – an interpretation status map. The TCC horizon and the Gold sequence boundary are “cut off” on some lines due to erosion. I suggest you use map 2 to record where the TCC horizon is present (with a symbol at the point where the red TOL horizon cuts it off. Similarly, you can use map 3 to record where the Gold horizon is present. (The cut off points for TCC and Gold are quite close to each other on each line where erosion has cut them off). |
| 2 | On the interpreted lines, an ‘x’ has been placed where the TCC and Gold horizons are interpreted to end. These termination points are interpretive (subjective) so treat them as “suggestions” only. If you consider it appropriate as a starting point, mark map 2 and map 3 with color where these horizons have been interpreted to be present and place a small “x” where they end, i.e., where they are interpreted to be cut off. Use a #2 pencil for the “x” labels so they can be easily erased, if necessary. NOTE: Due to the nature of seismic interference, it is never easy to know exactly where a stratigraphic interval thins to zero, whether by depositional or erosional thinning. |

Continued on next page

Exercise 32a:  Exploitation-Scale Mapping   continued

| Step | Action |
| --- | --- |
| 3 | All of the crosslines towards the north have interpretation. Use the line tie and horizon transfer method that you have now mastered to transfer the seven (7) horizons from the crosslines to the inlines. Complete the interpretation of each inline, at least for the northern half of the map. You have fewer constraints for the southern portions of the inlines, but extend the horizons as far as you can with at least a fair degree of confidence. |
| 4 | As time permits, work the horizons for the length of each inline and crossline. At some point you will be transferring your interpreted horizons from the inlines to the uninterpreted crosslines. Do you have any correlation busts? If you do, see if you can reinterpret the horizons so that they all tie consistently at each line intersection. |
| 5 | Once you have interpreted the horizons, work on the extent of the TCC horizon (base map 2) and the Gold horizon (base map 3). Can you draw a reasonable line on the map that marks where each of these horizons has been cut off? If your interpretation is internally consistent, then the map pattern of these cut off points should look geologically reasonable. |
| 6 | QUESTION: The interval between the TOL and TCC horizons is associated with the Gunard formation – a good seal. Where the TCC horizon is cut off, the Gunard formation would be absent (or at least below seismic resolution). How does this impact one of the “big 5” prospect elements: SEAL? |