Exercise 32b: Designing & Placing a Platform

| Objective | In this exercise, you will use the hydrocarbon (HC)-pore thickness map to answer four (4) early development questions based on some simple economic factors. |
| --- | --- |

| Materials | - Exercise instructions, including figures - Calculator - Tracing paper |
| --- | --- |

| Introduction | Since your team of geoscientists worked long and hard on the newly discovered Barracuda Field, you have been asked for your recommendations for the initial field development plan. You only need to consider geosciences issues along with simple economic assumptions. Others are looking into engineering, legal, taxation, and other non-geoscience issues. Here are the four (4) questions: 1. How many platforms do we need to maximize production while minimizing costs, so that we maximize profits: one, two or three? 2. What is the surface location for the platform or platforms? 3. How many production wells should we plan for each platform? 4. Where at the reservoir depth should we target each proposed production well? An important constraint is the “reach radius” for these platforms. We will assume that a well can target a location at the top of the reservoir that is anywhere within a radius of 2 km from the platform’s surface location. |
| --- | --- |

Continued on next page

Exercise 32b: Designing & Placing a Platform   continued

| Introduction continued | Another critical consideration is cost. The total costs (exploration, development, start-up, operations) must be less than the value of the gas produced. The table below lists our very simple cost factors: |
| --- | --- |

Simple cost factors:

Table 1

| Build and Install Platform with 1 Well Slot | $30 million |
| --- | --- |
| Enlarge Platform for Additional Well(s) | $2 million/well |
| Drilling Production Wells | $3 million/well |
| Platform Life Expectancy | 25 years |
| Platform Operations | $4 million/year |
| Well Operations | $0.1 million/year/well |
| Well Head Gas Price | $0.75/mmBTU |

Some necessary numbers and conversion factors:

Table 2

| Area of Each Cell on Map | 0.8 sq km |
| --- | --- |
| Recovery – Central Cell | 80% |
| Recovery – 6 neighboring Cells | 40% |
| Gas Expansion Factor | 111 scm/rcm |
| Energy | 36x106 mmBTU/km3 |
| Exploration Costs | $50 million |
| Development Costs | $75 million |

| Suggested Approach | Within your team, come up with two (2) or more possible production plans worth evaluating. Then have each team member run a cost analysis for one of these plans so that you have three (3) or four (4) possible production plans and the value and cost of each for comparison. Decide on which option is most appealing (biggest profit) and prepare a brief report on just that one case. |
| --- | --- |

Continued on next page

Exercise 25: Designing & Placing a Platform   continued

| Suggested Approach continued | For example, one plan might be a single platform with one well. For this case, you would position the platform above the cell with the largest HC pore thickness. One team member would first estimate the value of the gas in millions of dollars and then estimate the total costs; exploration, development, start-up (platform(s) & well(s)), and operating costs for 25 years. Value minus total costs would be the profit (or loss). The steps below will take you through: - The estimation of the value of the gas, - The costs associated with a development plan (N platforms with a total of M wells), and - The profit (or loss). |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | You will go through Steps 1 through 7 for each of your possible development plans (one per team member). For a particular plan, first decide on which cells you want to target as the central cell for a well. If all your targeted central cells are within a circle with a radius of 2 km or less, then one platform will suffice. If the targeted central cells are more widespread, then you will need more than one platform. You can use the tracing paper to copy the circle shown on Figure 1 that has a radius of 2 km and move it around the field to derive the surface location(s). Determine the number of platforms you will need and what the surface location(s) should be for each platform (i.e., the approximate cell addresses for each platform). NOTE: One way to work this exercise is to consider a one-platform case, a two-platform case, and perhaps a three-platform case. |

Continued on next page

Exercise 32b: Designing & Placing a Platform   continued

| Step | Action |
| --- | --- |
| 2 | Determine the volume of gas that each well will produce using the following method: a) Multiply the hydrocarbon (HC) pore thickness value of the central cell for the well by 0.8 (80% recovery efficiency). b) Add the HC pore thickness for each of the neighboring six (6) cells and multiply the sum by 0.4 (40% recovery efficiency). c) Add (2-a) and (2-b) and divide the sum by 1000. This gives the total HC pore thickness that the well can ‘drain’ in units of kilometers. d) Multiply (2-c) by 0.8 to get the gas volume in km3 at reservoir conditions (each cell is 0.8 km2) e) Multiply (2-d) by 111 to get the gas volume is km3 at surface conditions. Repeat these steps for each well in your plan and add all the gas volumes together. This will be the total gas volume. |
| 3 | Estimate the value of the gas: a) Multiple the total gas volume from Step 2 by 36x106 to get the energy in mmBTUs b) Multiply (3-a) by $0.75 to get the value of the gas in dollars. c) Divide (3-b) by 106 to get the total gas value in units of millions of dollars. |

Continued on next page

Exercise 32b: Designing & Placing a Platform   continued

| Step | Action |
| --- | --- |
| 4 | To determine start-up costs: a) Multiply the number of platforms in your plan by $30 million. b) If any of the platforms have more than one well, then add $2 million for each additional well on the platform to expand the platform’s capacity beyond that for a single well. c) Determine the total cost of the platforms: (4-a) plus the number of added wells times $2 million (4-b). d) Multiple the total number of wells by $3 million – the production well drilling cost per well. e) Add (4-c) and (4-d) to get the total start-up costs. |
| 5 | Next estimate the operation costs for 25 years: - Multiple the number of platforms by $100 million ($4 million /year * 25 years). - Multiply the number of wells by $2.5 million ($0.1 million /year * 25 years). - Add (5-a) and (5-b) to get the total operating costs. |
| 6 | Estimate the total costs (in units of $ million): - Exploration costs = $50 million - Production costs = $75 million - Start-up costs = _____________ (4-e) - Operation costs = _____________ (5-c) - Get the total costs = ________________ by adding (6-a), (6-b), (6-c) and (6-d) |

Continued on next page

Exercise 32b: Designing & Placing a Platform   continued

| Step | Action |
| --- | --- |
| 7 | Determine the estimated profit (or loss) for this particular development plan (in units of $ million): a) Total Gas Value = _____________ (3-c) b) Total Costs= _____________ (6-e) c) Profit = (7-a) minus (7-b) = _________________ |

Once each team member has run a possible development plan, look at the profit (loss) estimated for each. Which of the cases that you worked is most attractive financially?

As a team, prepare a short report. This should include:

1.	The surface location of the platform(s)

2.	The number of wells to be drilled from each platform

3.	The targeted (central) cell for each well

4.	The total gas value in $ million  (7-a)

5.	The total costs in $ million  (7-b)

6.	The estimated profit in $ million  (7-c)