Exercise 33a: Mapping a Gas Sand

| Objective | In this exercise, you will use several seismic lines to map the extent of the A-1 sand where it is filled with gas. The highly negative seismic amplitudes will be used to indicate the extent of the gas field. |
| --- | --- |

| Materials | - Exercise instructions - 6 inlines: 800, 820, 840, 860, 880, 900 - 5 crosslines: 420, 440, 460, 480, 500 - Pencils and a color pencil |
| --- | --- |

| Introduction | A well was drilled in 1972 to produce gas from a deep reservoir interval. While drilling, a 12 m gas sand was encountered. At this time, they were not able to evaluate the extent of the gas sand or how much gas could be recovered. So the A-1 sand was not completed (set up to be produced). In 1999, a 3D seismic survey was acquired to help production geoscientists manage the deeper field. The presence of gas in the A-1 sand gives it anomalously strong seismic amplitudes. This enabled seismic interpreters to map the areal extent of the A-1 gas for the first time. In exercise 33a, you will start to map the extent of gas in the A-1 sand. The impedance contrast (seismic amplitude response) between shale and the gas-charged A-1 sands is significantly greater than the impedance contrast between shale and the water-filled A-1 sands. Diagnostic large negative seismic amplitudes indicate the extent of the A-1 gas. In exercise 33b, you will estimate the quantity of gas that could be recovered from the A-1 sand. |
| --- | --- |

Continued on next page

Exercise 33a: Mapping a Gas Sand   			continued

| Step | Action |
| --- | --- |
| 1 | The A-1 horizon is interpreted for you on four (4) lines: Inlines 840 and 880 and Crosslines 420 and 480. A thicker, smaller dashed line marks the horizon where the trough is highly negative, due to gas in the pore space of the A-1 sand. Where the A-1 sand has brine in the pore space, the line is thinner and has longer dashes. Use a color pencil (or highlighter) to mark the extent of the gas on the seven (7) lines that do not have interpretation. |
| 2 | Mark on the base map, with a color pencil (one that can be erased), the extent of the gas on each of the four (4) lines that have been interpreted for you; e.g., on Inline 840 the gas extents from about the position on Crossline 448 to about the position of Crossline 510. |
| 3 | Using the basemap, pick a line that (1) has not been interpreted and (2) intersects a line that has been interpreted (e.g., Inline 860). Fold an interpreted line (Crossline 480) where it intersects the uninterpreted line (Inline 860). ‘Tie’ the A-1 sand horizon where the two (2) lines intersect, i.e., transfer the interpreted horizon to the corresponding trough on the uninterpreted line. Complete the ‘extent of gas’ interpretation (using the color pencil) on the newly-interpreted seismic line. |

Continued on next page

Exercise 33a: Mapping a Gas Sand   continued

| Step | Action |
| --- | --- |
| 4 | Mark the extent of gas on the basemap for the line you just interpreted. |
| 5 | Repeat the instructions in steps 3 and 4 until you have interpreted all or most of the uninterpreted seismic lines. |
| 6 | The interpretation process is highly iterative. You may find that your initial interpretation on a particular inline results in the extent of the gas being inconsistent at the position of one (or more) of the crosslines. Any inconsistencies must be resolved (fixed). If you have any inconsistencies, re-examine the lines and see if you can resolve the conflict in interpretation between the inline and the crossline. |
| 7 | Draw a polygon on the base map that encloses the extent of the gas based on your interpretation of the lines. This would be your interpretation of the areal extent of the A-1 gas field. IT IS NOT IMPORTANT THAT YOU GET A SATISFACTORY FINAL MAP. RATHER, IT IS IMPORTANT THAT YOU GRASP WHAT WE ARE DOING – THE PROCESS. |