Exercise 33b: Estimating Gas Recovery

| Objective | In this exercise, you will use a computer-generated map of seismic amplitude for the top of the A-1 sand and several assumed values to estimate the quantity of gas that can be recovered (produced) from the A-1 reservoir. |
| --- | --- |

| Materials | - Exercise instructions - Figures and computation sheet - Pencils and eraser |
| --- | --- |

| Introduction | This exercise is a continuation of Exercise 33a. You mapped the extent of the A-1 sand using eleven (11) lines out of a 3D seismic survey. A computer-generated seismic amplitude map provides a more detailed analysis of the extent of the gas. You will use this map to get an estimate of the area of the field. Then you will use a set of numbers, some from the original well and others based on reasonable estimates, to obtain the volume of gas we should expect to produce (get to the surface) if we were to drain this reservoir of its gas. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Figure 1 shows the extent of the gas in the A-1 sand that was obtained from a computer-generated seismic amplitude map. There is also a grid marking squares that are 200x200 meters in area. Your first task is to get the area of the gas field. Columns A, B and C have been done for you. Note that the units are in 100x100 m2 increments (a full cell = 4 units). Get a total for the remaining columns and sum the column totals to get the area of the gas field based on the amplitude map. |

Continued on next page

Exercise 33b: Estimating Gas Recovery   continued

| Step | Action |
| --- | --- |
| 2 | Since the units you used in step 1 are 100x100 m2, the area in km2 is the total you obtained in Step 1 multiplied by 0.01. Get the estimated area by performing this multiplication. Enter the area in km2 in row 1 of the computation sheet. |
| 3 | The A-1 sand was 12 m thick in the original well. The sand may be thicker in places; it is definitively thinner at the edges of the field. We will assume the field-wide average thickness is on the order of 10 meters (0.01 km). Enter this average thickness in km in row 2 of the computation sheet. |
| 4 | Two parameters determine the volume of the pores (spaces between sand grains): the net-to-gross and the porosity. We have values of both from logs from the original well and we will assume that these apply for the entire field. From the well logs, we estimate that the net-to-gross is 75% and the porosity is 17%. Multiply these two (2) numbers (0.75 * 0.17) and place the product in row 3. |
| 5 | Next we have to consider what portion of the pore space contains gas, as opposed to brine. From the well logs, we estimate the gas saturation is 90%. This is already on the computation sheet in row 4. |

Continued on next page

Exercise 33b: Estimating Gas Recovery   continued

| Step | Action |
| --- | --- |
| 6 | You are now ready to calculate the volume of gas in the reservoir at reservoir temperature and pressure. Simply multiply the values in rows 1, 2, 3, and 4. Enter the product in row 5 of the computation sheet. This is gas volume in km3. |
| 7 | We will assume a recovery efficiency of 50%. This means that we will only get ½ of the gas in the reservoir rocks. The recovery efficiency is already in row 6 of the computation sheet. |
| 8 | Multiply rows 5 and 6 and place the product in row 7. This is the volume of recoverable gas in km3 at reservoir conditions. We want gas volume recovered at the surface. The gas decreases in both temperature and pressure as it is brought up to the surface. Multiple the volume in row 7 by 73 (the volume expansion factor) and enter the product in row 8. |
| 9 | Many people in the energy industry like values of gas in units of cubic feet rather than cubic meters. Multiple row 8 by 35.3 to get the recoverable gas at surface conditions. Your final number will be gas volume in BCF (billion cubic feet). |